1 FES/FES2014

Two major steps using FES to generate tidal BC.

- ./gen_fg

The output will be fg.bp.

Remove the first two lines in fg.bp which will be used as input for next step.

- ./gen_harm_FES

ln –sf fg.bp open.ll

matlab –nodisplay

>>gen_harm_FES

The output will be ap.out.

Add the following lines (see how to generate them in “2 ADCIRC tidal BC”) to the ap.out and save ap.out as bctides.in:

07/27/2011 00:00:00 GMT

8 50. ntip

S2

2  0.112841  0.145444E-03  1.00000       0.00

M2

2  0.242334  0.140519E-03  1.00661      86.75

N2

2  0.046398  0.137880E-03  1.00661     198.72

K2

2  0.030704  0.145842E-03  0.96614     266.42

K1

1  0.141565  0.729212E-04  0.99458     223.23

P1

1  0.046843  0.725229E-04  1.00000     145.67

O1

1  0.100514  0.675977E-04  0.99104     219.40

Q1

1  0.019256  0.649585E-04  0.99104     331.37

8  nbfr

S2

0.145444E-03  1.00000       0.00

M2

0.140519E-03  1.00661      86.75

N2

0.137880E-03  1.00661     198.72

K2

0.145842E-03  0.96614     266.42

K1

0.729212E-04  0.99458     223.23

P1

0.725229E-04  1.00000     145.67

O1

0.675977E-04  0.99104     219.40

Q1

0.649585E-04  0.99104     331.37

1 !nope

748 3 0 0 0 !Ocean

Note:

- the first two columns if parameters for each tidal constituent are fixed, the last two columns of parameters are dependent on time, which can be generated by Nodal_Factors in ADCIRC. Will be described later.

- The order of the tidal constituents should be the same with that in the ap.out.

2 ADCIRC tidal BC

Cd /sciclone/home20/whuang07/schism10/NWM/Case1/RUN08c/ADCIRC%

Two steps using ADCIRC

- is to generate the amplitude and phase,

- is to generate the nodal factors for each tidal constituent.

Specitically:

- Under Amp_Phase

./gen_bc.pl hgrid.ll tides.out

- Under NodalFactor/ADCIRC

./tide_fac

The output will be tide_fac.out:

TIDAL FACTORS STARTING:  HR- 0.00,  DAY- 27,  MONTH-  7  YEAR- 2011

FOR A RUN LASTING    50.00 DAYS

CONST   NODE     EQ ARG (ref GM)

NAME   FACTOR    (DEG)

K1    0.99458     223.23

O1    0.99104     219.40

P1    1.00000     145.67

Q1    0.99104     331.37

N2    1.00661     198.72

M2    1.00661      86.75

S2    1.00000       0.00

K2    0.96614     266.42

- Add the two factors for each tidal constituent in bctide.in (see highlighted below)

07/27/2011 00:00:00 GMT

7 50. ntip

S2

2  0.112841  0.145444E-03  1.00000       0.00

M2

2  0.242334  0.140519E-03  1.00661      86.75

N2

2  0.046398  0.137880E-03  1.00661     198.72

K2

2  0.030704  0.145842E-03  0.96614     266.42

K1

1  0.141565  0.729212E-04  0.99458     223.23

O1

1  0.100514  0.675977E-04  0.99104     219.40

Q1

1  0.019256  0.649585E-04  0.99104     331.37

7  nbfr

S2

0.145444E-03  1.00000       0.00

M2

0.140519E-03  1.00661      86.75

N2

0.137880E-03  1.00661     198.72

K2

0.145842E-03  0.96614     266.42

K1

0.729212E-04  0.99458     223.23

O1

0.675977E-04  0.99104     219.40

Q1

0.649585E-04  0.99104     331.37

1 !nope

- 3 0 0 !Ocean

- Add output from the 1st step to bctides