# SELFE NetCDF output description

# Introduction

The purpose of the work is to output the SELFE simulation results in NetCDF format.

# What the subroutines do?

The NetCDF program does the following things:

- Output the SELFE results into single NetCDF format based on the NetCDF conversion.(not like the  original SELFE output, one processor generate one file)

- The rank 0 processor will do the following things:

  - Collect the data from other process

  - Assemble the collected data with the right order (based on nodes and vertical level)

  - Output the data to the NetCDF file

- The other processors send the data to the rank 0 processors, these process will not do output process.

This procedure is proved as the most efficient way to output the results.

# Required package

## Windows:

- Windows binary version of netcdf version: 3.6.1. (already in the package)

(need to put it in the appropriate location )

- MPI library, currently MSMPI is used. Mpich2  is also possible

- Windows version of “METISlib.lib” and “ParMetisLib.li”. (already in the package)

## Linux :

The same as original SELFE except need to use the netcdf library.

The folder_linux is used for linux operation system.

# Subroutine components:

The package provides the windows version project, which can only run under the release module and the make file for Linux to run:

- data_type_mod.F90: define the data type used.

- check_error.F90: define the NetCDF check error part and also the used module.

NOTE:

- For windows: need to use the module “netcdf90” and “funMaxModule”

- For Linux : comment the “netcdf90” and “funMaxModule” and use “netcdf”

- nsf_container.F90: pack the NetCDF model

- nsf_self_vars.F90: define the variables used for NetCDF output

- nsf_selfe_out_8.FI: the file included in the SELFE CODE for output

# How to use the NetCDF output subroutines

- Use the “use nsf_self_vars_mod” in the main program.

- Include the “nsf_selfe_out_8.FI” at the output part “IF(iwrite == 0 .OR. iwrite ==1 ) THEN       ”

- In param.in if iwrite =1 : original selfe output, if iwrite =2 netcdf output

# How to compile in Linux

The program is tested on CETO, set the variables in CETO and type make.

Don’t forget to change the used models in file “check_error.F90”

# Details of the subroutines

- nsf_selfe_nectcdf_out_init: init the netcdf output by define the variables and so on.

- nfs_self_create_file: create the netcdf file, define the array and properties.

- nsf_selfe_out_grid: output the SELFE grid to netCDF file (not time dependent)

- nfs_collect_rdata_2d: gather the scalar data in netcdf;

- nfs_collect_rdata_3d: gather the 3D scalar data in netcdf;

# Netcdf output benchmark:

- # of element 149196 , # of nodes 79786, # of vertical layer: 31. Processors: 32

- Output variables: elev, vert, temp, salt, hvel;

- Output data size/step: ~66M

- Output time : <5s;

- The time consumption for output is in filer “mirror.out” , for example:

start to write data to netcdf fileoutputs/1.nc

Finished to output netcdf file:

Write data to netcdf file takes:       3.30seconds